Heaven's Light is Our Guide

Rajshahi University of Engineering & Technology

Department of Computer Science & Engineering

Course No.: CSE 2103  |  Course Title: Digital Logic Design

Assignment — Question 1: Optimized Counter Design with Constraints

# Problem Statement

Design a synchronous MOD-12 counter that follows the custom non-binary sequence given below:

0 → 3 → 7 → 11 → 2 → 5 → 9 → 1 → 6 → 10 → 4 → 8 → (repeat)

The counter is a MOD-12 counter; in other words, it is a 4-bit synchronous counter capable of traversing the given random sequence. The four state bits are Q₁, Q₂, Q₃, Q₄, where Q₁ is the Most Significant Bit (MSB) and Q₄ is the Least Significant Bit (LSB).

# Task 1: State Diagram and State Table

## 1.1 State Diagram

The state diagram depicts the cyclic transition of the counter through all 12 valid states. Each state is represented by its 4-bit binary equivalent, and directed arcs indicate the clock-triggered transitions between consecutive states.

0000 → 0011 → 0111 → 1011 → 0010 → 0101 → 1001 → 0001 → 0110 → 1010 → 0100 → 1000 → (repeat)

Figure: State Diagram of the MOD-12 Custom Sequence Counter

## 1.2 State Table

The state table below lists all 16 possible 4-bit combinations. The 12 valid states follow the defined sequence, while the 4 invalid states (highlighted in yellow) are forced to return to State 0 (0000) on the next clock cycle, implementing the self-correction mechanism.

| Current State |  |  |  | Next State |  |  |  | Flip-Flop Inputs (D FF) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q₁ | Q₂ | Q₃ | Q₄ | Q₁⁺ | Q₂⁺ | Q₃⁺ | Q₄⁺ | D₁ | D₂ | D₃ | D₄ |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 1: State Table for MOD-12 Custom Sequence Counter (Yellow rows = Invalid states)

# Task 2: Flip-Flop Selection and Justification

The selected flip-flop type for this design is the D Flip-Flop. For a D flip-flop, the excitation requirement is the simplest possible: the next state value is directly applied as the flip-flop input, i.e., D = Q⁺. This makes the flip-flop input equations identical to the next-state equations.

## 2.1 Power Consumption

The D flip-flop consumes less power compared to the JK flip-flop because it has a simpler internal gate structure. The JK flip-flop requires additional feedback logic, which increases switching activity and power dissipation. In a synchronous design where all flip-flops are clocked simultaneously, the reduced gate count of D flip-flops directly translates to lower dynamic power consumption.

## 2.2 Circuit Complexity

The D flip-flop requires the simplest excitation: the next state value is directly applied as input. This significantly simplifies the combinational logic design. In contrast, the JK flip-flop requires separate J and K excitation equations for each bit, and the T flip-flop demands XOR-based logic that adds considerable complexity for non-toggle sequences such as this custom counter.

## 2.3 Speed (Propagation Delay)

The D flip-flop exhibits lower propagation delay because its input logic is straightforward with no feedback path. For a synchronous counter where all flip-flops are clocked simultaneously, minimizing propagation delay ensures reliable high-speed operation within a single clock period.

# Task 3: Excitation Equations and K-Map Minimization

Since the D flip-flop input is identical to the desired next state (Dᵢ = Qᵢ⁺), the excitation equations are derived directly from the state table. Karnaugh maps (K-maps) are applied to minimize each excitation function using don't-care conditions for all four invalid states (states 12–15, where Q₁Q₂ = 11).

## 3.1 Minimized Excitation Equations

After applying K-map minimization, the minimized Boolean expressions for each D flip-flop input are:

D₁ = Q̅₁ · Q₂

D₂ = Q̅₁Q̅₂Q₄ + Q̅₂Q₃Q̅₄

D₃ = Q̅₁Q̅₂Q̅₃ + Q̅₁Q₂Q₃ + Q̅₂Q₃Q₄

D₄ = Q₁Q̅₂Q̅₃Q₄ + Q̅₁Q̅₂Q̅₄ + Q̅₁Q₂Q₄ + Q̅₁Q₃Q₄

## 3.2 K-Map Derivation Summary

Each 4-variable K-map is populated using Q₁Q₂ as row variables and Q₃Q₄ as column variables in standard Gray code order (00, 01, 11, 10). The four invalid states (Q₁Q₂ = 11) are treated as don't-care entries (X) to allow maximum grouping and minimal logic.

- D₁ K-map: A single group of four 1-cells in the Q₁Q₂ = 01 row yields D₁ = Q̅₁Q₂.

- D₂ K-map: Two groups are identified — a group of three 1-cells in the Q₃Q₄ = 01/11/10 columns of the 00-row, and one isolated 1-cell in the 10-row at Q₃Q₄ = 10, yielding D₂ = Q̅₁Q̅₂Q₄ + Q̅₂Q₃Q̅₄.

- D₃ K-map: Three prime implicant groups cover all minterms, yielding D₃ = Q̅₁Q̅₂Q̅₃ + Q̅₁Q₂Q₃ + Q̅₂Q₃Q₄.

- D₄ K-map: Four groups are required to cover all minterms, yielding the four-term expression for D₄.

# Task 4: Complete Circuit Design (Logisim Implementation)

The complete synchronous MOD-12 counter circuit was implemented in Logisim using four D flip-flops labeled Q₁ (MSB) through Q₄ (LSB), and a combinational excitation logic network derived from the minimized Boolean expressions above. All flip-flops share a common clock signal, confirming the synchronous operation.

The combinational block implements D₁, D₂, D₃, D₄ using standard AND, OR, and NOT gates. The circuit faithfully produces the correct state transition at every rising clock edge, cycling through the 12 valid states in the prescribed sequence.

[Figure: MOD-12 Counter Design for the Given Sequence — Logisim Screenshot]

[Figure: Timing Diagram of the MOD-12 Counter — Logisim Waveform]

# Task 5: Invalid State Handling (Self-Correction Mechanism)

The designed counter employs 4 flip-flops, yielding 2⁴ = 16 possible binary state combinations. However, only 12 of these are utilized in the valid counting sequence (states 0 through 11). The remaining 4 states are invalid:

| State | Binary Representation | Next State (Forced) |
| --- | --- | --- |
| 12 | 1100 | 0000 (State 0) |
| 13 | 1101 | 0000 (State 0) |
| 14 | 1110 | 0000 (State 0) |
| 15 | 1111 | 0000 (State 0) |

Table 2: Invalid States and Their Forced Next-State Transitions

During power-up or due to electrical noise, the counter may accidentally enter one of these invalid states. To handle this robustly, all invalid states are forced to transition to State 0 (0000) on the next clock cycle.

From the excitation equations, it is observed that whenever Q₁Q₂ = 11 (i.e., any of the four invalid states), all flip-flop inputs D₁, D₂, D₃, D₄ evaluate to 0. Consequently, on the next clock edge, the counter unconditionally transitions to 0000. This built-in self-correction mechanism ensures that no invalid state will cause a permanent fault or erroneous sequence in the counter operation.

# Task 6: Analysis and Comparison

## 6.1 Fan-Out Analysis

Fan-out refers to the maximum number of gate inputs that a single gate output can reliably drive while maintaining valid logic levels without signal degradation. In this MOD-12 counter, each flip-flop output feeds directly into the combinational excitation logic. Inspecting the excitation equations:

| Flip-Flop Output | Used In Equations | Fan-Out Demand |
| --- | --- | --- |
| Q₁ | D₁, D₂, D₃, D₄ | High |
| Q₂ | D₁, D₂, D₃, D₄ | High |
| Q₃ | D₂, D₃, D₄ | Medium |
| Q₄ | D₂, D₃, D₄ | Medium |

Table 3: Fan-Out Demand per Flip-Flop Output

Q₁ and Q₂ appear in all four excitation equations, creating high fan-out demands on these outputs. In a physical CMOS implementation, buffering may be required for Q₁ and Q₂ outputs to prevent signal degradation. Q₃ and Q₄, appearing in only three equations each, present moderate fan-out loads.

## 6.2 Propagation Delay

Propagation delay is the time elapsed between a change at the input and the corresponding change at the output of a logic gate or flip-flop. In a synchronous counter, the combinational logic must complete its computation within one clock period.

- Among all excitation circuits, D₂ and D₄ contain the most gate levels, making them the critical paths of the circuit.

- The D flip-flop contributes the lowest delay because it uses direct assignment (D = Q⁺), requiring no intermediate state conversion logic.

- The maximum operating frequency is constrained by: fₘₐˣ = 1 / (tₚᵉ + tₜ₟ₜ + tₜₚᵈ), where tₚᵉ is the combinational propagation delay, tₜ₟ₜ is the clock-to-Q delay of the flip-flop, and tₜₚᵈ is the setup time.

## 6.3 Power Dissipation Trade-Offs

Dynamic power dissipation in CMOS logic is directly proportional to switching activity and gate count, given by P = α · Cₗ · Vᴵᴵ² · f, where α is the activity factor.

- D flip-flops use the least combinational logic due to their direct-assignment nature, minimizing the activity factor α and total capacitance Cₗ.

- JK flip-flops contain internal feedback gates that switch even during no-change conditions, creating unnecessary switching activity and increased power dissipation.

- T flip-flops require XOR-based logic, which adds complexity and higher switching activity for non-toggle sequences such as this custom counter.

Overall, the D flip-flop offers the best balance of low power consumption, minimal circuit complexity, and fast propagation for this custom MOD-12 synchronous counter design.

# Design Justification Report

The design of the synchronous MOD-12 custom sequence counter satisfies all requirements specified in the assignment. The following summarizes the key design decisions:

- Flip-Flop Selection: D flip-flops were selected over JK and T alternatives based on their superior performance across all three evaluation criteria: power consumption, circuit complexity, and propagation delay.

- State Encoding: Natural binary encoding is used, with Q₁ as MSB and Q₄ as LSB, allowing direct implementation in Logisim and straightforward K-map analysis.

- Excitation Logic Minimization: Karnaugh maps were applied with don't-care entries for the four invalid states, producing minimal Sum-of-Products (SOP) expressions that reduce gate count and interconnection.

- Invalid State Handling: The self-correction mechanism is inherent in the minimized excitation equations — whenever Q₁Q₂ = 11, all D inputs evaluate to 0, forcing an unconditional return to State 0 on the next clock edge.

- Circuit Implementation: The complete circuit was implemented and verified in Logisim, confirming correct state transitions across all 12 valid states and self-correction from all 4 invalid states.

- Fan-Out Management: Q₁ and Q₂ exhibit high fan-out demand; in a physical implementation, output buffers would be recommended for these signals to maintain signal integrity.

— End of Assignment —